DC Chopper control vs. Phase Angle control for appliance and power Tool Applications--Who wins?

Jeff Shoemaker, Renesas Electronics America Inc.

3/30/2011 05:35 PM EDT

Engineers working with universal motors for equipment like power tools, vacuum cleaners and mixers have to decide between control methods for this type of motor. The two most common motor-control methods are: DC Chopper control and Phase Angle control. This article explores the differences between the two methods, identifying some of the key trade-offs that can be useful for motor-control designers when determining which is the best control method to use for their specific application.

Shown in Figures 1 and 6, respectively, the triac control and chopper control circuits used to drive the universal motor can be used in a very simple household appliance.  The main difference is that the triac and Gate Drive Transistor in the Phase Angle control circuit are replaced by an IGBT.  The chopper circuit also introduces in a full-wave rectifier to the AC line circuit because the IGBT only conducts current in one direction...

The main principle behind the Phase Angle control circuit is that the microcontroller (MCU) is used to vary the firing angle to the triac.  By adjusting the firing angle to the triac, i.e., the point in the AC voltage waveform where the triac turns on, the voltage applied to the motor can be modulated and the speed of the motor can be regulated.  The firing angle of the triac is referenced to the zero crossing of the AC waveform, which is sensed using a zero crossing detect circuit on the line voltage.  The MCU uses a special timer that emits a series of one-shot pulses into the gate of the triac to turn it on.  A series of pulses are recommended as opposed to just a single pulse per cycle, because the motor is an inductive load, and a pulse train ensures the triac conducts current in both positive and negative directions.

An illustration of this principle can be seen in Figures 2 and 3.  In Figure 2, the gate pulses to the triac are for roughly one third of the duty cycle of the voltage waveform.  The triac is conducting during this time.  In the laboratory, where this data was collected, this combination of voltage and firing angle resulted in a motor speed of 900 RPM.   In Figure 3, there are more gate pulses resulting in a larger firing angle and a longer period in which the triac is conducting.  With this firing angle, the resulting motor speed is 16,000 RPM.

Since the AC voltage waveform is 60 Hz, it is worth noting that the firing angle is applied 120 times a second, because there are two zero crossings per cycle.  There is no linear relationship between torque and AC current, so a closed-loop control scheme for controlling torque is not practical, nor does it make sense in this case.

In the laboratory tests, an R8C MCU from Renesas Electronics was used to implement the control algorithm for both control methods. As noted in Figure 4, the device offers a number of 8-bit and 16-bit timers that can be configured to perform a variety of functions such as triac and chopper control, for instance matching to triacs rates at 20 and 25 amps for phase-angle circuit or to IGBTs rated at 45 and 60 amps for DC chopper control.

Timer RB of the Renesas MCU is an 8-bit timer with an 8-bit pre-scalar that can implement a general-purpose timer or waveform generator.   The general purpose timer is used to determine the time between two consecutive Hall sensor signals to determine the speed.  Timer RC is a 16-bit timer which can be configured as an input capture, output compare or PWM generator.    This timer is used to generate required PWM with accurate timings to fire the TRIAC or the DC Chopper.  Timer RD is an advanced 16-bit timer that can generate PWM individually out of each of the output pins or generate up to three complementary PWM pairs with automatic dead time insertion.  This timer is typically used to control the H-bridge implementation (equivalent of DC Chopper circuit) when the motor is also rotated in both directions.  In this article, motor is rotating only in one direction and therefore no H-bridge is used.

The speed of the universal motor can be controlled open-loop or closed-loop using a speed sensor. Figure 5 shows an illustration of the control algorithm for the Phase Angle control scheme implementing closed-loop speed control. The speed measurement is implemented with Hall sensors, which are connected to interrupt pins on the MCU. The timer RB can be configured as a general-purpose 16-bit timer that, upon interrupt, takes a count reading from the pulse outputs of the sensors. Based on these readings and the timing, the firmware calculates the measured speed. For triac control, another timer interrupt occurs when a zero crossing is detected. Timer RB or RC can count the time for when the triac is off, or the "wait time". Based on the desired and measured speed, the PI compensator can adjust the wait time.  When the wait time is expired, pulse-width modulation (PWM) pulses are applied to the triac gates to turn them on for the remainder of the half-wave cycle.  The PWM timer RC/RD fires after the wait time has expired, generating a string of 1ms output pulses to the gate of the Triac.  This "Pulse Train" timer is terminated on each zero crossing.

It should be no surprise then that the PWM Chopper circuit works quite a bit differently.  As shown in Figure 6, Instead of applying an AC waveform to the motor, the line voltage is rectified with the full-bridge rectifier.  This is to ensure that the IGBT is always in the forward-biased region.  The gate is driven by a PWM signal generated by the MCU that switches the IGBT on and off at 12KHz or higher.  Since the current waveform is a DC quantity, there exists a linear relationship between current and torque.  This means that the current can be measured, and a PI loop can be applied to control current and thus control torque.  A second PI loop can be used to measure the speed and control the speed of the motor as well.

The DC chopper control scheme does not measure zero crossings, as was the case in phase angle control.  The phase angle method control output is restricted to changing 120 times per cycle.  Again, the quantities that we are dealing with here are in the DC domain, which means that the DC chopper method does not share these restrictions.  Therefore, much higher control frequencies can be realized, resulting in a much wider dynamic control range.

Another key difference between the two control methods is that the DC chopper method requires only a single PWM channel for the IGBT- one that continuously varies the PWM Duty Cycle, 0% to 100%, of the 12 KHz gate drive signal.  .  When using the Phase Angle control method, two PWM channels are needed in order to capture the firing angle for the entire cycle.

As illustrated in Figure 7, with the DC chopper algorithm, hall sensors are used with timer RB to measure the speed.  This time, the PI loop outputs a current command as its control output.  Again, this is because the current is a DC quantity, where there exists a linear and predictable relationship between current and torque.  Using the MCU's A/D converter, designers can measure the current from the shunt resistor so that the current loop can be closed.  The current loop also uses PI control and outputs PWM counts to the Timer RB/RC PWM register.  The PWM outputs turns the MOSFET on and off at 12 KHz or higher.