;====================================================================

;

; Author        : ADI - Apps

;

; Date          : July 2003

;

; File          : UART.asm

;

; Hardware      : ADuC845

;

; Description   : This Program transmits the numbers 0->7F (starting

;                 with 0) down the UART in ASCII form to the PC where

;                 they can be viewed using the preconfigured

;                 Hyperterminal program.  (c:\ADuC\uart.ht)

;                 After the transmission of the 16 bytes a 5 second

;                 delay is called and the process is repeated.

;

;====================================================================

;

$MOD845                                 ;Use ADuC845 predefined Symbols

LED	EQU	P3.4

;____________________________________________________________________

; BEGINNING OF CODE

CSEG

ORG 0000H

JMP MAIN

ORG 0060H                    ; Start code at address above interrupts

MAIN:                          ; Main program

;        MOV 	PLLCON, #03h

MOV 	PLLCON, #00h

MOV	   	T2CON, #034h

MOV	   	RCAP2H, #FFh

MOV	   	RCAP2L, #AEh

;        MOV     T3CON,#83h

MOV     T3CON,#00h

;        MOV     T3FD,#12h

MOV     SCON,#52h

;		MOV     SCON,#50h

MOV		TH2,#0

MOV		TL2,#0

START:

CPL      LED           ; CPL LED with each transmission

MOV      DPTR, #TITLE

CALL     SENDSTRING    ; write title block on screen

MOV      R0, #00H    ; Start transmissions from 0

MOV      R1, #08H    ; Start a new line after 8 transmissions

LOOP1:                        ; Every eight transmissions start on a

; new line

MOV      A, #10       ; Transmit a linefeed

CALL     SENDCHAR

MOV      A, #13       ; Transmit a carriage return

CALL     SENDCHAR

MOV      R1, #08H

LOOP2:

MOV      A, #20H      ; Transmit a SPACE (=ASCII 20) between

; transmissions on same line

CALL     SENDCHAR

MOV      A, R0        ; Transmit R0 = present data

CALL     SENDVAL

INC      R0           ; increment data

CJNE     A, #7FH, CONT ; check if data =7F, if no continue

JMP      WAIT5S        ; if = 7F wait  and repeat

CONT:   DEC      R1            ; decrement R1....

MOV      A, R1

CJNE     A, #00H, LOOP2 ; and check if new line is required

; jump to loop 2 for a space

JMP      LOOP1          ; jump to loop 1 for a new line

WAIT5S: MOV      A, #60         ; wait

CALL     DELAY

JMP      START        ; start transmissions again

;____________________________________________________________________

; SENDSTRING

SENDSTRING:     ; sends ASCII string to UART starting at location

; DPTR and ending with a null (0) value

PUSH    ACC

PUSH    B

CLR     A

MOV     B,A

IO0010: MOV     A,B

INC     B

MOVC    A,@A+DPTR

JZ      IO0020

CALL    SENDCHAR

JMP     IO0010

IO0020: POP     B

POP     ACC

RET

;____________________________________________________________________

; SENDCHAR

SENDCHAR:       ; sends ASCII value contained in A to UART

JNB     TI,$            ; wait til present char gone

CLR     TI              ; must clear TI

MOV     SBUF,A

RET

;____________________________________________________________________

; SENDVAL

SENDVAL:        ; converts the hex value of A into two ASCII chars,

; and then spits these two characters up the UART.

; does not change the value of A.

PUSH    ACC

SWAP    A

CALL    HEX2ASCII

CALL    SENDCHAR        ; send high nibble

POP     ACC

PUSH    ACC

CALL    HEX2ASCII

CALL    SENDCHAR        ; send low nibble

POP     ACC

RET

;____________________________________________________________________

; HEX2ASCII

HEX2ASCII:      ; converts A into the hex character representing the

; value of A's least significant nibble

ANL     A,#00Fh

CJNE    A,#00Ah,$+3

JC      IO0030

ADD     A,#007h

IO0030: ADD     A,#'0'

RET

;____________________________________________________________________

; DELAY

DELAY:

MOV	R2,A	   ; Acc holds delay variable

DLY0:	  MOV	R3,#255     ; Set up delay loop0

DLY1:	  MOV	R4,#255    ; Set up delay loop1

DJNZ	R4,$	   ; Dec R4 & Jump here until R4 is 0

DJNZ	R3,DLY1    ; Dec R3 & Jump DLY1 until R3 is 0

DJNZ	R2,DLY0    ; Dec R2 & Jump DLY0 until R2 is 0

RET		   ; Return from subroutine

;____________________________________________________________________

TITLE:    DB 10,10,13,'____________________________________',10,13

DB 'Analog Devices MicroConverter ADuC845',10,13

DB '         UART Demo Routine',10,13

DB '  Transmission of Data from 0 to 7F',10,13,0

END