// File:	MAIN.C (CAN-RS232 MONITOR)

// Title:	The .... ...... .......

// Version:	1.4

// Last updated:	29.06.18

// Author:	GM

// Target:	Olimex P103 board with STM32F103RBT6 (8.0000 MHz quartz)

// Support E-mail:	.....@mail.ru

//#include <stdio.h>

#include "STM32F10x\stm32f10x_gpio.h"

#include "STM32F10x\stm32f10x_nvic.h"

u16 varray[256];

u32 vaverage,sigma;

const float astep=0.81030273;

vu32 msg01[4],msg02[4];

u16 vold,vnew,vout;

vu8 iv,k,uflg,rflg0,rflg1,aflg;

vu8 rxbuf[8];

vu8 xrate,xmode,xnode;

void ADC_IRQHandler(void);

void CAN_SCE_IRQHandler(void);

void USB_HP_CAN_TX_IRQHandler(void);

void USART1_IRQHandler(void);	//extern

void USB_LP_CAN_RX0_IRQHandler(void)

{

GPIOC->BSRR=0x10000000;	//stat led on pc12=0

if(CAN->RF0R & 0x03)	//pending msg in fifo0?

{

msg01[0]=CAN->sFIFOMailBox[0].RIR;

msg01[1]=CAN->sFIFOMailBox[0].RDTR;

msg01[2]=CAN->sFIFOMailBox[0].RDLR;

msg01[3]=CAN->sFIFOMailBox[0].RDHR;

CAN->RF0R |=0x00000020;	//release the message from fifo0

rflg0=1;

}

}

void CAN_RX1_IRQHandler(void)

{

if(CAN->RF1R & 0x03)	//pending message in fifo1?

{

msg02[0]=CAN->sFIFOMailBox[1].RIR;

msg02[1]=CAN->sFIFOMailBox[1].RDTR;

msg02[2]=CAN->sFIFOMailBox[1].RDLR;

msg02[3]=CAN->sFIFOMailBox[1].RDHR;

CAN->RF1R |=0x00000020;	//release the message from fifo1

rflg1=1;

}

}

int main (void)

{

vu32 i1,CANID2;

vu16 i,j,k;

vu16 j0,j1,j2;

vu16 k0,k1,k2,k3,k4;

vs16 m,va;

vu8 RATE,MODE,NODE,cmd;

RCC->CR |= 0x00010000;	//HSEON

RCC->CFGR |= 0x04054002;	//sysCLK->MCO,adcpre=4,sys=PLLCLK

RCC->APB1ENR |=0x12000001;	//PWREN,CANEN,TIM2EN

RCC->APB2ENR |=0x0000521C;	//USART1EN,SPI1EN,ADC1,IOP-CBA-EN

RCC->CR |= 0x01010000;	//PLLON,HSEON

while(((RCC->CR) & 0x02000000)==0);	//wait of PLLRDY

RCC->APB2ENR |=0x00000001;	//AFIOEN-afio clock enable

GPIOA->CRL =0xD4D600A4;	//7-miso,5-clk1,4-cs,1-OC2,0-button

GPIOA->CRH =0x000004AB;	//10-9 alt RX1-TX1,8-alt50

GPIOB->CRL =0x00000001;	//pb0-output10

GPIOB->CRH =0x000000B4;	//pb9-pb8 CAN Tx-Rx

GPIOC->CRL =0x00000000;	//adc10-pc0

GPIOC->CRH =0x00020000;	//STAT LED pc12

AFIO->MAPR = 0x00004000;	//CANTX-CANRX mapped to pb9-pb8

CAN->MCR=0x00000001;	//init mode

CAN->BTR=0x006F0003;	//¬LBKM,1+7+16/4=6MHz

CAN->MCR=0x00000000;	//normal mode

CAN->sTxMailBox[0].TDLR=0x0000;	//low data3-data0

CAN->sTxMailBox[0].TDHR=0x0000;	//high data7-data4

CAN->sTxMailBox[0].TDTR=0x0008;	//data size

//-------------adjust the rx mail box0-------------

CAN->FMR=0x0101;	//ini mode for filters

CAN->FM1R=0x0000;	//mask filter mode

CAN->FS1R=0x0003; 	//32-bit scale

CAN->FFA1R=0x0000;	//fifo0 assignment

CAN->FA1R=0x0003;	//FB1,FB0 is active

CAN->sFilterRegister[0].FR1=(0x280<<21); //id0 RPDO2

CAN->sFilterRegister[0].FR2=(0x000<<21); //id0 mask

CAN->FMR=0x0100;	//no finit

CAN->IER=0x00000012;	//FMPIE1,FMPIE0,¬TMEIE

//---------------------------------------------------

USART1->BRR=0x0D0;	//rate=24002.1(16*13)=115.395 kbaud

USART1->CR1=0x200C;	//UE,TE,RE

// USART1->CR1 |= 0x0020;	//RXNEIE=1

//---------------------------------------------------

//  TIM2->CR1=0x0081;	//APRE=1,CEN=1

//  TIM2->CCMR1=0x3000;	//toggle OC2REF in 2 ms

//  TIM2->CCER=0x0010;	//CC2Enable on PA1

//  TIM2->CNT=0x0000; 	//to be on the safe side

//  TIM2->PSC=240;	//24000/120=200 kHz (5us)

//  TIM2->ARR=12500;	//200000/400=500 Hz (2 ms)

//  TIM2->CCR2=200;	//value to compare

//---------------------------------------------------

//  ADC1->SQR1 =0x00000000;	//one conversion

//  ADC1->SMPR2 =0x00000001;	//1-7,5 cycles (0.54 us)

//  ADC1->SQR3 =0x0000000A;	//ADC10-pc0

//  ADC1->CR1 =0x00000020;	//EOCIE

//  ADC1->CR2 =0x00160005;	//EXTTRIG,CC2,CAL,ADON

SPI1->CR1=0x0B4C;	//DFF,SSM,SSI,SPE,PCLK/4,MSTR

// NVIC->ISER[0] |=(1<<(ADC1_2_IRQChannel & 0x1F));	//ADC1 & ADC2 global interrupt

// NVIC->ISER[0] |=(1<<(USB_HP_CAN_TX_IRQChannel & 0x1F)); //CAN TX interrupts

NVIC->ISER[0] |=(1<<(USB_LP_CAN_RX0_IRQChannel & 0x1F)); //CAN RX0 interrupts

NVIC->ISER[0] |=(1<<(CAN_RX1_IRQChannel & 0x1F));	//CAN RX1 interrupts

// NVIC->ISER[0] |=(1<<(CAN_SCE_IRQChannel & 0x1F));	//CAN SCE interrupts

// NVIC->ISER[0] |=(1<<(USART1_IRQChannel & 0x1F));	 //0x25 USART1 global interrupt

k=0;

uflg=1;

RATE=4;

MODE=2;

NODE=0x3E;

vaverage=0;

cmd=0;

while(1)

{

if(((USART1->SR) & 0x20)) cmd=USART1->DR;

if(cmd==0x08)

{

while(((USART1->SR) & 0x20)==0);

NODE=USART1->DR;

while(((USART1->SR) & 0x20)==0);

MODE=USART1->DR;

CANID2=(0x300+NODE)<<21;	//CAN-ID for request of adc data

CAN->sTxMailBox[0].TDLR=(cmd<<24)+(NODE<<16)+(MODE<<8)+RATE;	//low data3-data0

CAN->sTxMailBox[0].TDHR=0;	//high data7-data4

CAN->sTxMailBox[0].TDTR=0x08;	//data size

CAN->sTxMailBox[0].TIR=CANID2+1;	//STID,IDE,TXRQ

while(((CAN->TSR) & 0x0001) == 0);	//until transmission completed

CAN->TSR |= 1;	//clear flags of mb0

cmd=0;

}

if(rflg0==1)

{

while(((USART1->SR) & 0x80)==0);

USART1->DR=msg01[0] >>29;	//msb STID[10-8]

while(((USART1->SR) & 0x80)==0);

USART1->DR=msg01[0] >>21;	//lsb STID[7-0]

while(((USART1->SR) & 0x80)==0);

USART1->DR=msg01[2] >>24;	//D3

while((USART1->SR & 0x80)==0);

USART1->DR=msg01[2]>>16;	//D2

while((USART1->SR & 0x80)==0);

USART1->DR=msg01[2] >>8;	//D1=msb vout

while((USART1->SR & 0x80)==0);

USART1->DR=msg01[2];	//D0=lsb vout

while((USART1->SR & 0x80)==0);

USART1->DR=msg01[3] >>24;	//D7

while((USART1->SR & 0x80)==0);

USART1->DR=msg01[3]>>16;	//D6

while((USART1->SR & 0x80)==0);

USART1->DR=msg01[3] >>8;	//D5

while((USART1->SR & 0x80)==0);

USART1->DR=msg01[3];	//D4

rflg0=0;

}

}

}