# Using the Communications Control

The Communications control allows you to add both simple serial port communication functionality to your application and advanced functionality to create a full-featured, event-driven communications tool.

The Communications Control

The Communications control provides an interface to a standard set of communications commands. It allows you to establish a connection to a serial port, connect to another communication device (a modem, for instance), issue commands, exchange data, and monitor and respond to various events and errors that may be encountered during a serial connection.

## Possible Uses

- To dial a phone number.

- To monitor a serial port for incoming data.

- To create a full-featured terminal program.

## Sample Applications: Dialer.vbp and VBTerm.vbp

The Dialer.vbp and VBTerm.vbp sample applications which are listed in the samples directory, demonstrate simple and complex (respectively) programming techniques of the Communications control.

## Basics of Serial Communications

Every computer comes with one or more serial ports. They are named successively: COM1, COM2, and so on. On a standard PC, the mouse is usually connected to the COM1 port. A modem may be connected to COM2, a scanner to COM3, etc. Serial ports provide a channel for the transmission of data from these external serial devices.

The essential function of the serial port is to act as an interpreter between the CPU and the serial device. As data is sent through the serial port from the CPU, Byte values are converted to serial bits. When data is received, serial bits are converted to Byte values.

A further layer of interpretation is needed to complete the transmission of data. On the operating system side, Windows uses a communications driver, Comm.drv, to send and receive data using standard Windows API functions. The serial device manufacturer provides a driver that connects its hardware to Windows. When you use the Communications control, you are issuing API functions, which are then interpreted by Comm.drv and passed to the device driver.

As a programmer, you need only concern yourself with the Windows side of this interaction. As a Visual Basic programmer, you need only concern yourself with the interface that the Communications control provides to API functions of the Windows communications driver. In other words, you set and monitor properties and events of the Communications control.

## Establishing the Serial Connection

The first step in using the Communications control is establishing the connection to the serial port. The following table lists the properties that are used to establish the serial connection:

| Properties | Description |
| --- | --- |
| CommPort | Sets and returns the communications port number. |
| Settings | Sets and returns the baud rate, parity, data bits, and stop bits as a string. |
| PortOpen | Sets and returns the state of a communications port. Also opens and closes a port. |

### Opening the Serial Port

To open a serial port, use the CommPort, PortOpen, and Settings properties. For example:

' Open the serial port

MSComm1.CommPort = 2

MSComm1.Settings = "9600,N,8,1"

MSComm1.PortOpen = True

The CommPort property sets which serial port to open. Assuming that a modem is connected to COM2, the above example sets the value to 2 (COM2) and connects to the modem. You can set the CommPort property value to any number between 1 and 16 (the default is 1). If, however, you set this value to a COM port that does not exist for the system on which your application is run, an error will be generated.

The Settings property allows you to specify the baud rate, parity, and the number of data bits and stop bits. By default, the baud rate is set at 9600. The parity setting is for data validation. It is commonly not used, and set to "N". The data bits setting specifies the number of bits that represent a chunk of data. The stop bit indicates when a chunk of data has been received.

Once you’ve specified which port to open and how data communication is to be handled, you use the PortOpen property to establish the connection. It is a Boolean value, True or False. If, however, the port is not functional, if the CommPort property is set incorrectly, or if the device does not support the settings you’ve specified, an error will be generated or the external device may not work correctly. Setting the value of the PortOpen property to False closes the port.

## Working with a Modem

In most cases, you will use the Communications control to program your application to work with a modem. With the Communications control, you can use the standard Hayes-compatible command set to dial a phone number, or connect to and interact with another modem.

Once the serial port connection has been established using the CommPort, Settings, and PortOpen properties, you use the Output property to activate and interact with the modem. The Output property is used to issue commands which control the interaction between one modem and another. For example:

' Activate the modem and dial a phone number.

MSComm1.Output = "ATDT 555-5555" & vbCr

In the example above, the command "AT" initiates the connection, "D" dials the number, and "T" specifies touch tone (rather than pulse). A carriage return character (vbCr) must be specified when outputting to a terminal. You do not need to add the return character when outputting byte arrays.

When a command is successfully processed, an "OK" result code will be returned. You can test for this result code to determine if a command was processed successfully.

For More Information   For a complete list of Hayes-compatible commands, check your modem documentation.

## Setting Receive and Transmit Buffer Properties at Design Time

When a port is opened, receive and transmit buffers are created. To manage these buffers, the Communications control provides you with a number of properties that can be set at design time using the control’s Property Pages.

Setting buffer properties at design time

### Buffer Memory Allocation

The InBufferSize and OutBufferSize properties specify how much memory is allocated to the receive and transmit buffers. Each are set by default to the values shown above. The larger you make the number, the less memory you have available to your application. If, however, your buffer is too small, you run the risk of overflowing the buffer unless you use handshaking.

Note   Given the amount of memory available to most PCs at this time, buffer memory allocation is less crucial because you have more resources available. In other words, you can set the buffer values higher without affecting the performance of your application.

### The RThreshold and SThreshold Properties

The RThreshold and SThreshold properties set or return the number of characters that are received into the receive and transmit buffers before the OnComm event is fired. The OnComm event is used to monitor and respond to changes in communications states. Setting the value for each property to zero (0) prevents the OnComm event from firing. Setting the value to something other than 0 (1, for instance) causes the OnComm event to be fired every time a single character is received into either buffer.

For More Information   See "The OnComm Event and the CommEvent Property" in this topic for more information on these properties.

### The InputLen and EOFEnable Properties

Setting the InputLen property to 0 causes the Communications control to read the entire contents of the receive buffer when the Input property is used. When reading data from a machine whose output is formatted in fixed-length blocks of data, the value of this property can be set appropriately.

The EOFEnable property is used to indicate when an End of File (EOF) character is found during data input. Setting this to True causes data input to stop and the OnComm event to fire to inform you that this condition has occurred.

For More Information   See "Managing the Receive and Transmit Buffers" and "The OnComm Event and the CommEvent Property" in this topic for more information.

## Managing the Receive and Transmit Buffers

As mentioned above, receive and transmit buffers are created whenever a port is opened. The receive and transmit buffers are used to store incoming data and to transmit outgoing data. The Communications control allows you to manage these buffers by providing you with a number of properties that are used to place and retrieve data, return the size of each buffer, and handle both text and binary data. Properly managing these buffers is an important part of using the Communications control.

### The Receive Buffer

The Input property is used to store and retrieve data from the receive buffer. For example, if you wanted to retrieve data from the receive buffer and display it in a text box, you might use the following code:

txtDisplay.Text = MSComm1.Input

To retrieve the entire contents of the receive buffer, however, you must first set the InputLen property to 0. This can be done at design or run time.

You can also receive incoming data as either text or binary data by setting the InputMode property to one of the following Visual Basic constants: comInputModeText or comInputModeBinary. The data will either be retrieved as string or as binary data in a Byte array. Use comInputModeText for data that uses the ANSI character set and comInputModeBinary for all other data, such as data that has embedded control characters, Nulls, etc.

As each byte of data is received, it is moved into the receive buffer and the InBufferCount property is incremented by one. The InBufferCount property, then, can be used to retrieve the number of bytes in the receive buffer. You can also clear the receive buffer by setting the value of this property to 0.

### The Transmit Buffer

The Output property is used to send commands and data to the transmit buffer.

Like the Input property, data can be transmitted as either text or binary data. The Output property, however, must transmit either text or binary data by specifying either a string or Byte array variant.

You can send commands, text strings, or Byte array data with the Output property. For example:

' Send an AT command

MSComm1.Output = "ATDT 555-5555"

' Send a text string

MsComm1.Output = "This is a text string"

' Send Byte array data

MSComm1.Output = Out

As previously mentioned, transmit lines must end with a carriage return character (vbCr). In the last example, Out is a variable defined as a Byte array: Dim Out() As Byte. If it were a string variant, it would be defined as: Dim Out() As String.

You can monitor the number of bytes in the transmit buffer by using the OutBufferCount property. You can clear the transmit buffer by setting this value to 0.

## Handshaking

An integral part of managing the receive and transmit buffers is ensuring that the back-and-forth transmission of data is successful — that the speed at which the data is being received does not overflow the buffer limits, for example.

Handshaking refers to the internal communications protocol by which data is transferred from the hardware port to the receive buffer. When a character of data arrives at the serial port, the communications device has to move it into the receive buffer so that your program can read it. A handshaking protocol ensures that data is not lost due to a buffer overrun, where data arrives at the port too quickly for the communications device to move the data into the receive buffer.

You set the Handshaking property to specify the handshaking protocol to be used by your application. By default, this value is set to none (comNone). You can, however, specify one of the other following protocols:

| Setting | Value | Description |
| --- | --- | --- |
| comNone | 0 | No handshaking (Default). |
| comXOnXOff | 1 | XOn/XOff handshaking. |
| comRTS | 2 | RTS/CTS (Request To Send/Clear To Send) handshaking. |
| comRTSXOnXOff | 3 | Both Request To Send and XOn/XOff handshaking. |

The protocol that you choose depends upon the device to which you’re connecting. Setting this value to comRTSXOnXOff supports both of the protocols.

In many cases, the communications protocol itself handles handshaking. Therefore, setting this property to something other than comNone may result in conflicts.

Note   If you do set this value to either comRTS or comRTSXOnXOff, you need to set the RTSEnabled property to True. Otherwise, you will be able to connect and send, but not receive, data.

## The OnComm Event and the CommEvent Property

Depending upon the scope and functionality of your application, you may need to monitor and respond to any number of events or errors which may occur during the connection to another device or in the receipt or transmission of data.

The OnComm event and the CommEvent property allow you to trap and check the value of communication events and errors.

When a communication event or error occurs, the OnComm event is fired and the value of the CommEvent property is changed. Therefore, if necessary, you can check the value of the CommEvent property each time the OnComm event is fired. Because communications (especially over telephone lines) can be unpredictable, trapping these events and errors allows you to respond to them appropriately.

The following table lists the communication events that will trigger the OnComm event. The values will then be written to the CommEvent property.

| Constant | Value | Description |
| --- | --- | --- |
| comEvSend | 1 | There are fewer than SThreshold number of characters in the transmit buffer. |
| comEvReceive | 2 | Received RThreshold number of characters. This event is generated continuously until you use the Input property to remove the data from the receive buffer. |
| comEvCTS | 3 | Change in Clear To Send line. |
| comEvDSR | 4 | Change in Data Set Ready line. This event is only fired when DSR changes from 1 to 0. |
| comEvCD | 5 | Change in Carrier Detect line. |
| comEvRing | 6 | Ring detected. Some UARTs (universal asynchronous receiver-transmitters) may not support this event. |
| comEvEOF | 7 | End Of File (ASCII character 26) character received. |

The OnComm event is also triggered, and a value is written to the CommEvent property, when the following errors are encountered.

| Setting | Value | Description |
| --- | --- | --- |
| comEventBreak | 1001 | A Break signal was received. |
| comEventFrame | 1004 | Framing Error. The hardware detected a framing error. |
| comEventOverrun | 1006 | Port Overrun. A character was not read from the hardware before the next character arrived and was lost. |
| comEventRxOver | 1008 | Receive Buffer Overflow. There is no room in the receive buffer. |
| comEventRxParity | 1009 | Parity Error. The hardware detected a parity error. |
| comEventTxFull | 1010 | Transmit Buffer Full. The transmit buffer was full while trying to queue a character. |
| comEventDCB | 1011 | Unexpected error retrieving Device Control Block (DCB) for the port. |

# HOWTO: Use MSCOMM32.OCX to Transfer Data on DBCS Windows ID: Q158008. The information in this article applies to: - Microsoft Visual Basic Control Creation, Learning, Professional, and Enterprise Editions for Windows, version 5.0 - Microsoft Visual Basic Professional and Enterprise Editions, 16-bit and 32-bit, for Windows, version 4.0 ## SUMMARY This article illustrates some techniques using the MSComm32.OCX control for receiving and transmitting binary data on double-byte character set (DBCS) Windows. ## MORE INFORMATION The Communications control, MSCOMM32.OCX, provides serial communications for your application by allowing the transmission and reception of data through a serial port where only a string of characters to the transmit buffer is permitted. This feature limits you to transmitting only text- based files. The binary data must be converted one byte at a time to a character and then transmitted. When received, the data must be converted from a character back into binary data one byte at a time. See the REFERENCES section below for more information. On double-byte character set Windows, such as Korean, Japanese, and Chinese Windows, binary data will be corrupted if one of the binary values matches a DBCS lead character. The MSComm control will interpret this byte and the following byte as one double-byte character and return only one byte for the equivalent ASCII character. To resolve this problem, which is specific to DBCS Windows, you can implement certain encoding algorithms that can convert binary data into text data and vice versa. This article demonstrates how to convert one byte binary data into ASCII size of three characters to preserve the lead character. On the receiving end, these three ASCII characters are converted back into one-byte binary data. For example: ASC('a') = 97 ' 97 would be transmitted as three characters: '0''9''7' NOTE: The sample code is reading the input file as binary. It is not necessary to convert each character to ASCII value. If you use a modem instead of RS-232 cable, you need to add some codes to establish the modem connection. For example: - MSComm1.Output = 'ATDT 999-9999' for Transmitter and - - MSComm1.Output = 'AT' for Receiver Once the connection is established, everything should work the same way. WARNING: USE OF THE SAMPLE CODE PROVIDED IN THIS ARTICLE IS AT YOUR OWN RISK. Microsoft provides this sample code "as is" without warranty of any kind, either express or implied, including but not limited to the implied warranties of merchantability and/or fitness for a particular purpose. ### Sample Code - Transmitter - Start Visual Basic. If Visual Basic is already running, from the File menu, choose New Project. Form1 will be created by default. - Place Microsoft Comm Control on Form1. If Microsoft Comm Control cannot be found in the control bar, go to Tools menu, choose Custom Controls, and then check the Microsoft Comm Control on the List Box. - Add the following code to the General Declarations section of Form1: - ' General Declaration section - Dim Offset As Long - Dim FileData As Byte - Dim FileName As String - Add the following code to the Load event of Form1: - ' Load event of Form - ' 28800 baud, no parity, 8 data, and 1 stop bit. - ' In RS-232, maximum speed of 28800 baud can be used - Private Sub Form_Load() - MSComm1.Settings = "28800,N,8,1" - MSComm1.InputLen = 1 - MSComm1.CommPort = 2 - MSComm1.PortOpen = True - ' Change the file path to point to the file you want to transfer - FileName = "d:\sample.hlp" - Offset = 1 - Open FileName For Binary Access Read As #1 - End Sub - Add the following code to the OnComm event of MSComm1: - ' ONCOMM event of Comm OLE Control - ' The following code is needed to make sure that next set of - ' characters can be transmitted (CommEvent = 2 is triggered from the - ' receiver side) - Private Sub MSComm1_OnComm() - If MSComm1.CommEvent = 2 Then - temp = MSComm1.Input - If Offset <= FileLen(FileName) Then - Get #1, Offset, FileData - q = FileData - MSComm1.Output = Format(q, "000") - Offset = Offset + 1 - End If - End If - End Sub - Add the following code to the UnLoad event of Form1: - ' UnLoad event of Form - Private Sub Form_Unload(Cancel As Integer) - Close #1 - End Sub - Make sure that the Properties of MSComm1 are changed as follows: - RThreshold = 1 ' triggers when at least one char is on the buffer - SThreshold = 3 ### Sample Code - Receiver - Start Visual Basic. If Visual Basic is already running, from the File menu, choose New Project. Form1 will be created by default. - Place Microsoft Comm Control on Form1. If Microsoft Comm Control cannot be found in the control bar, go to Tools menu, choose Custom Controls, and then check the Microsoft Comm Control on the List Box. - Add a CommandButton to Form1. Command1 is created by default. - Add the following code to the General Declarations section of Form1: - ' General Declaration section - Dim ByteCount As Long - Dim FileData As Byte - Add the following code to the Load event of Form1: - ' Load event of Form - ' 28800 baud, no parity, 8 data, and 1 stop bit. - Private Sub Form_Load() - MSComm1.Settings = "28800,N,8,1" - MSComm1.InputLen = 3 - MSComm1.CommPort = 1 - MSComm1.PortOpen = True - ' Change the file path to point to the file you want to store - Open "c:\sample.txt" For Binary Access Write As #1 - End Sub - Add the following code to the OnComm event of MSComm1: - ' OnComm event - Private Sub MSComm1_OnComm() - If MSComm1.CommEvent = 2 And MSComm1.InBufferCount > 0 Then - FileData = CInt(MSComm1.Input) - ByteCount = ByteCount + 1 - Put #1, ByteCount, FileData - MSComm1.Output = Chr$(26) - End If - End Sub - Add the following code to the Click event of Command1: - ' CLICK event of button - Private Sub Command1_Click() - MSComm1.Output = Chr$(26) - End Sub - Add the following code to the UnLoad event of Form1: - ' UnLoad even of Form - Private Sub Form_Unload(Cancel As Integer) - Close #1 - End Sub - Make sure that the Properties of MSComm1 are changed as follows: RThreshold = 3 The Transmitter and Receiver codes should be compiled separately and placed on two separate machines where one machine is the designated transmitter and the other one is the receiver. The event-driven technique generates an OnComm event when there are characters waiting in the input buffer. Also, the CommEvent property will contain a numeric 2. For the OnComm event to be triggered, you must set the Rthreshold property to a value other than its default of zero. The most common setting for the RThreshold property is 1, meaning that the OnComm event is triggered if a minimum of one character is waiting in the input buffer. In this article, the numeric 3 is being used as a value of RThreshold property because 3 characters at a time are being sent. ## REFERENCES For more information on how to use the MSComm control to transmit and receive binary data on US Windows, please see the following article in the Microsoft Knowledge Base: Q151899 : Transmitting and Receiving Binary Data with MSComm Control For additional information, please see the following article in the Microsoft Knowledge Base: Q194923 : HOWTO: Trap Control Characters Using the MSComm Control Additional query words: Comm KBTRADCH KBSIMCH KBKOREA KBJAPAN kbVBp500 kbVBp600 kbVBp kbdsd kbDSupport kbCtrl Keywords : Version : WINDOWS:4.0,5.0 Platform : WINDOWS Issue type :

Last Reviewed: May 21, 1999

# INFO: Receiving Data Using the MSComm Control's OnComm Event ID: Q194922. The information in this article applies to: - Microsoft Visual Basic Learning, Professional, and Enterprise Editions for Windows, versions 5.0, 6.0 ## SUMMARY This article demonstrates a basic template for using an event driven data receiving routine with the OnComm event. This is the recommended method of receiving all types of data using the MSComm control. For information on converting binary data, please see the REFERENCES section at the end of this article. ## MORE INFORMATION When using the MSCOMM control to send and receive serial data, it is best to use event driven communications. This method is more efficient than polling, as it uses less CPU time and will only initiate an action when input is received. Below is a simple template to use which will allow for very efficient receiving of characters with minimal data loss. The idea is to use the OnComm event to receive the data, then write the raw data to a string and pass it to another routine to do the processing of the raw data. Passing the data to another routine allows the OnComm event to receive every event without being "suppressed," while the actual parsing of the string takes place elsewhere. The following example will assume that you have a null modem cable attached between COM1 and COM2. This is only necessary for testing. App1 is the COM1 application, App2 is the COM2 application. ### Steps to Create App1 - Create a new Standard EXE project. Form1 is created by default. - Choose Components from the Project menu, check the "Microsoft Comm Control," and click OK. - Add an MSCOMM control to the form. - Add a TextBox and a CommandButton to the form. Change the MultiLine property of the TextBox to True. - Add the following code to Form1's code window: - Private Sub Form_Load() - Form1.Caption = "App1" - With MSCOMM1 - .Handshaking = 2 - comRTS - .RThreshold = 1 - .RTSEnable = True - .Settings = "9600,n,8,1" - .SThreshold = 1 - .PortOpen = True - ' Leave all other settings as default values. - End With - Command1.Caption = "&Send" - Text1.Text = "Test string from App1 " - End Sub - Private Sub Command1_Click() - MSComm1.Output = Text1.Text - End Sub - Private Sub Form_Unload(Cancel As Integer) - MSComm1.PortOpen = False - End Sub ### Steps to Create App2 - Start a new instance of Visual Basic. - Create a new Standard EXE project. Form1 is created by default. - Choose Components from the Project menu, check the "Microsoft Comm Control," and click OK. - Add an MSCOMM control to the form. - Add a TextBox to the form. Change the MultiLine property of the TextBox to True. Enlarge the TextBox so it will cover most of the form, as you will be displaying all the received data in it. - Add the following code to Form1's code window: - Private Sub Form_Load() - Form1.Caption = "App2" - With MSComm1 - .CommPort = 2 - .Handshaking = 2 - comRTS - .RThreshold = 1 - .RTSEnable = True - .Settings = "9600,n,8,1" - .SThreshold = 1 - .PortOpen = True - ' Leave all other settings as default values. - End With - Text1.Text = "" - End Sub - Private Sub Form_Unload(Cancel As Integer) - MSComm1.PortOpen = False - End Sub - Private Sub MSComm1_OnComm() - Dim InBuff As String - Select Case MSComm1.CommEvent - ' Handle each event or error by placing - ' code below each case statement. - ' This template is found in the Example - ' section of the OnComm event Help topic - ' in VB Help. - ' Errors - Case comEventBreak ' A Break was received. - Case comEventCDTO ' CD (RLSD) Timeout. - Case comEventCTSTO ' CTS Timeout. - Case comEventDSRTO ' DSR Timeout. - Case comEventFrame ' Framing Error. - Case comEventOverrun ' Data Lost. - Case comEventRxOver ' Receive buffer overflow. - Case comEventRxParity ' Parity Error. - Case comEventTxFull ' Transmit buffer full. - Case comEventDCB ' Unexpected error retrieving DCB] - ' Events - Case comEvCD ' Change in the CD line. - Case comEvCTS ' Change in the CTS line. - Case comEvDSR ' Change in the DSR line. - Case comEvRing ' Change in the Ring Indicator. - Case comEvReceive ' Received RThreshold # of chars. - InBuff = MSComm1.Input - Call HandleInput(InBuff) - Case comEvSend ' There are SThreshold number of - ' characters in the transmit buffer. - Case comEvEOF ' An EOF character was found in the - ' input stream. - End Select - End Sub - Sub HandleInput(InBuff As String) - ' This is where you will process your input. This - ' includes trapping characters, parsing strings, - ' separating data fields, etc. For this case, you - ' are simply going to display the data in the TextBox. - Text1.SelStart = Len(Text1.Text) - Text1.SelText = InBuff - End Sub ### Steps to Run Applications - Press the F5 key or click the run button on each project. You will need to move the apps around so you can see them both running at the same time. - Make sure you have a standard null modem cable between COM1 and COM2, or change the MSComm1.Port properties to match the comm ports you are using. - Click the CommandButton on App1; you should see the string "Test string from App1" appear in App2's TextBox for each button click. ## REFERENCES For information on converting binary data, please see the following articles in the Microsoft Knowledge Base: Q151899 : Transmitting and Receiving Binary Data with MSComm Control Q158008 : HOWTO: Use MSCOMM32.OCX to Transfer Data on DBCS Windows For additional information, please see the following article in the Microsoft Knowledge Base: Q194923 : HOWTO: Trap Control Characters Using the MSComm Control Additional query words: mscomm Keywords : kbIO kbVBp kbVBp500 kbVBp600 kbCodeSam Version : WINDOWS:5.0,6.0 Platform : WINDOWS Issue type : kbinfo

Last Reviewed: May 14, 1999

# MSCOMM32.OCX MSComm Control The MSComm control (MSCOMM32.OCX) comes with Visual Basic. This one came with VB 6.0. We have a license to distribute this OCX with our Visual Basic application. We have tried in to get a clarification from Microsoft if it is OK to distribute stand alone. We asked: "Do we have the right to distribute the MSComm control which comes with Visual Basic with an Access application?" When this question was asked of the MS Access technical support people we could not find anyone that knew what the MSComm control is. The closest we got was someone that thought we were talking about the Common Dialog control. It is our assumption that because we have the right to distribute this with a VB application we also should have the right to distribute it stand-alone. This is the MSCOMM32.OCX that should be stored in your \windows\system directory. To register within Access go to TOOLS/ACTIVE X CONTROLS and select \windows\system\mscomm32.ocx or Microsoft Communications Control and click on the REGISTER button. If you get a licensing error the only way we know to get the licensing is to install VB 6 Professional. You can try the registry entry listed below. Microsoft Technical Support and Sales Staff was of no use when we tried find out how to register or purchase the Comm control. We were sent this string to be entered into the registry to clear the license issue. We have not tried it but the sender says it worked for them. [HKEY_CLASSES_ROOT\Licenses\4250E830-6AC2-11cf-8ADB-00AA00C00905] @ = "kjljvjjjoquqmjjjvpqqkqmqykypoqjquoun" Download Now mscomm.zip 50K bytes

|  | The MSComm control provides serial communications for your application by allowing the transmission and reception of data through a serial port. Syntax MSComm Remarks The MSComm control provides the following two ways for handling communications: - Event-driven communications is a very powerful method for handling serial port interactions. In many situations you want to be notified the moment an event takes place, such as when a character arrives or a change occurs in the Carrier Detect (CD) or Request To Send (RTS) lines. In such cases, use the MSComm control's OnComm event to trap and handle these communications events. The OnComm event also detects and handles communications errors. For a list of all possible events and communications errors, see the CommEvent property. - You can also poll for events and errors by checking the value of the CommEvent property after each critical function of your program. This may be preferable if your application is small and self-contained. For example, if you are writing a simple phone dialer, it may not make sense to generate an event after receiving every character, because the only characters you plan to receive are the OK response from the modem. Each MSComm control you use corresponds to one serial port. If you need to access more than one serial port in your application, you must use more than one MSComm control. The port address and interrupt address can be changed from the Windows Control Panel. Although the MSComm control has many important properties, there are a few that you should be familiar with first. Properties; Description. CommPort; Sets and returns the communications port number.. Settings; Sets and returns the baud rate, parity, data bits, and stop bits as a string.. PortOpen; Sets and returns the state of a communications port. Also opens and closes a port.. Input; Returns and removes characters from the receive buffer.. Output; Writes a string of characters to the transmit buffer.. # OnComm Event The OnComm event is generated whenever the value of the CommEvent property changes, indicating that either a communication event or an error occurred. Syntax Private Sub object_OnComm () The OnComm event syntax has these parts: Part; Description. object; An object expression that evaluates to an object in the Applies To list.. Remarks The CommEvent property contains the numeric code of the actual error or event that generated the OnComm event. Note that setting the RThreshold or SThreshold properties to 0 disables trapping for the comEvReceive and comEvSend events, respectively. # CommPort Property Sets and returns the communications port number. Syntax object.CommPort[ = value ] The CommPort property syntax has these parts: Part; Description. object; An object expression that evaluates to an object in the Applies To list.. value; A integer value specifying the port number.. Remarks You can set value to any number between 1 and 16 at design time (the default is 1). However, the MSComm control generates error 68 (Device unavailable) if the port does not exist when you attempt to open it with the PortOpen property. Warning You must set the CommPort property before opening the port. Data Type Integer # Handshaking Property Sets and returns the hardware handshaking protocol. Syntax object.Handshaking [ = value ] The Handshaking property syntax has these parts: Part; Description. object; An object expression that evaluates to an object in the Applies To list.. value; An integer expression specifying the handshaking protocol, as described in Settings.. Settings The settings for value are: Setting; Value; Description. comNone; 0; (Default) No handshaking.. comXOnXOff; 1; XON/XOFF handshaking.. comRTS; 2; RTS/CTS (Request To Send/Clear To Send) handshaking.. comRTSXOnXOff; 3; Both Request To Send and XON/XOFF handshaking.. Remarks Handshaking refers to the internal communications protocol by which data is transferred from the hardware port to the receive buffer. When a character of data arrives at the serial port, the communications device has to move it into the receive buffer so that your program can read it. If there is no receive buffer and your program is expected to read every character directly from the hardware, you will probably lose data because the characters can arrive very quickly. A handshaking protocol insures data is not lost due to a buffer overrun, where data arrives at the port too quickly for the communications device to move the data into the receive buffer. Data Type Integer # RThreshold Property Sets and returns the number of characters to receive before the MSComm control sets the CommEvent property to comEvReceive and generates the OnComm event. Syntax object.Rthreshold [ = value ] The Rthreshold property syntax has these parts: Part; Description. object; An object expression that evaluates to an object in the Applies To list.. value; An integer expression specifying the number of characters to receive before generating the OnComm event.. Remarks Setting the RThreshold property to 0 (the default) disables generating the OnComm event when characters are received. Setting RThreshold to 1, for example, causes the MSComm control to generate the OnComm event every time a single character is placed in the receive buffer. Data Type Integer # Settings Property Sets and returns the baud rate, parity, data bit, and stop bit parameters. Syntax object.Settings [ = value ] The Settings property syntax has these parts: Part; Description. object; An object expression that evaluates to an object in the Applies To list.. value; An string expression representing the communications port settings, as described below.. Remarks If value is not valid when the port is opened, the MSComm control generates error 380 (Invalid property value). Value is composed of four settings and has the following format: "BBBB,P,D,S" Where BBBB is the baud rate, P is the parity, D is the number of data bits, and S is the number of stop bits. The default value of value is: "9600,N,8,1" The following table lists the valid baud rates. Setting 110. 300. 600. 1200. 2400. 9600 (Default). 14400. 19200. 28800. 38400. 56000. 128000. 256000. The following table describes the valid parity values. Setting; Description. E; Even. M; Mark. N; (Default) None. O; Odd. S; Space. The following table lists the valid data bit values. Setting 4. 5. 6. 7. 8 (Default). The following table lists the valid stop bit values. Setting. 1; (Default). 1.5. 2. Data Type String # InputLen Property Sets and returns the number of characters the Input property reads from the receive buffer. Syntax object.InputLen [ = value ] The InputLen property syntax has these parts: Part; Description. object; An object expression that evaluates to an object in the Applies To list.. value; An integer expression specifying the number of characters the Input property reads from the receive buffer.. Remarks The default value for the InputLen property is 0. Setting InputLen to 0 causes the MSComm control to read the entire contents of the receive buffer when Input is used. If InputLen characters are not available in the receive buffer, the Input property returns a zero-length string (""). The user can optionally check the InBufferCount property to determine if the required number of characters are present before using Input. This property is useful when reading data from a machine whose output is formatted in fixed-length blocks of data. Data Type Integer # Input Property Returns and removes a stream of data from the receive buffer. This property is not available at design time and is read-only at run time. Syntax object.Input The Input property syntax has these parts: Part; Description. object; An object expression that evaluates to an object in the Applies To list.. Remarks The InputLen property determines the number of characters that are read by the Input property. Setting InputLen to 0 causes the Input property to read the entire contents of the receive buffer. The InputMode property determines the type of data that is retrieved with the Input property. If InputMode is set to comInputModeText then the Input property returns text data in a Variant. If InputMode is comInputModeBinary then the Input property returns binary data in an array of bytes in a Variant. Data Type Variant # PortOpen Property Sets and returns the state of the communications port (open or closed). Not available at design time. Syntax object.PortOpen [ = value ] The PortOpen property syntax has these parts: Part; Description. object; An object expression that evaluates to an object in the Applies To list.. value; A boolean expression specifying the state of the communications port.. Settings The settings for value are: Setting; Description. True; Port is opened. False; Port is closed. Remarks Setting the PortOpen property to True opens the port. Setting it to False closes the port and clears the receive and transmit buffers. The MSComm control automatically closes the serial port when your application is terminated. Make sure the CommPort property is set to a valid port number before opening the port. If the CommPort property is set to an invalid port number when you try to open the port, the MSComm control generates error 68 (Device unavailable). In addition, your serial port device must support the current values in the Settings property. If the Settings property contains communications settings that your hardware does not support, your hardware may not work correctly. If either the DTREnable or the RTSEnable properties is set to True before the port is opened, the properties are set to False when the port is closed. Otherwise, the DTR and RTS lines remain in their previous state. Data Type Boolean # DTREnable Property Determines whether to enable the Data Terminal Ready (DTR) line during communications. Typically, the Data Terminal Ready signal is sent by a computer to its modem to indicate that the computer is ready to accept incoming transmission. Syntax object.DTREnable[ = value ] The DTREnable property syntax has these parts: Part; Description. object; An object expression that evaluates to an object in the Applies To list.. Value; A Boolean expression specifying whether to enable the Data Terminal Ready (DTR) line, as described in Settings.. Settings The settings for value are: Setting; Description. True; Enable the Data Terminal Ready line.. False; (Default) Disable the Data Terminal Ready line.. Remarks When DTREnable is set to True, the Data Terminal Ready line is set to high (on) when the port is opened, and low (off) when the port is closed. When DTREnable is set to False, the Data Terminal Ready always remains low. Note In most cases, setting the Data Terminal Ready line to low hangs up the telephone. Data Type Boolean # RTSEnable Property Determines whether to enable the Request To Send (RTS) line. Typically, the Request To Send signal that requests permission to transmit data is sent from a computer to its attached modem. Syntax object.RTSEnable[ = value ] The RTSEnable property syntax has these parts: Part; Description. object; An object expression that evaluates to an object in the Applies To list.. value; An boolean expression specifying whether the Request To Send (RTS) line is enabled, as described in Settings.. Settings The settings for value are: Setting; Description. True; Enables the Request To Send line.. False; (Default) Disables the Request To Send line.. Remarks When RTSEnable is set to True, the Request To Send line is set to high (on) when the port is opened, and low (off) when the port is closed. The Request To Send line is used in RTS/CTS hardware handshaking. The RTSEnable property allows you to manually poll the Request To Send line if you need to determine its state. For more information on handshaking protocols, see the Handshaking property. Data Type Boolean # CommEvent Property Returns the most recent communication event or error. This property is not available at design time and is read-only at run time. Syntax object.CommEvent The CommEvent property syntax has these parts: Part; Description. object; An object expression that evaluates to an object in the Applies To list.. Remarks Although the OnComm event is generated whenever a communication error or event occurs, the CommEvent property holds the numeric code for that error or event. To determine the actual error or event that caused the OnComm event, you must reference the CommEvent property. The CommEvent property returns one of the following values for communication errors or events. These constants can also be found in the Object Library for this control. Communication errors include the following settings: Constant; Value; Description. comEventBreak; 1001; A Break signal was received.. comEventFrame; 1004; Framing Error. The hardware detected a framing error.. comEventOverrun; 1006; Port Overrun. A character was not read from the hardware before the next character arrived and was lost.. comEventRxOver; 1008; Receive Buffer Overflow. There is no room in the receive buffer.. comEventRxParity; 1009; Parity Error. The hardware detected a parity error.. comEventTxFull; 1010; Transmit Buffer Full. The transmit buffer was full while trying to queue a character.. comEventDCB; 1011; Unexpected error retrieving Device Control Block (DCB) for the port.. Communications events include the following settings: Constant; Value; Description. comEvSend; 1; There are fewer than Sthreshold number of characters in the transmit buffer.. comEvReceive; 2; Received Rthreshold number of characters. This event is generated continuously until you use the Input property to remove the data from the receive buffer.. comEvCTS; 3; Change in Clear To Send line.. comEvDSR; 4; Change in Data Set Ready line. This event is only fired when DSR changes from 1 to 0.. comEvCD; 5; Change in Carrier Detect line.. comEvRing; 6; Ring detected. Some UARTs (universal asynchronous receiver-transmitters) may not support this event.. comEvEOF; 7; End Of File (ASCII character 26) character received.. Data Type Integer # Output Property Writes a stream of data to the transmit buffer. This property is not available at design time and is write-only at run time. Syntax object.Output [ = value ] The Output property syntax has these parts: Part; Description. object; An object expression that evaluates to an object in the Applies To list.. value; A string of characters to write to the transmit buffer.. Remarks The Output property can transmit text data or binary data. To send text data using the Output property, you must specify a Variant that contains a string. To send binary data, you must pass a Variant which contains a byte array to the Output property. Normally, if you are sending an ANSI string to an application, you can send it as text data. If you have data that contains embedded control characters, Null characters, etc., then you will want to pass it as binary data. Data Type Variant |
| --- | --- |