# Dallas 1-Wire(R) Master Common files File Documentation

## OWIcrc.c File Reference

### Detailed Description

Atmel Corporation

- File: OWIcrc.c

- Compiler: IAR EWAAVR 3.20a

- Support mail: avr@atmel.com

- Supported devices: All AVRs.

- Application Note: AVR318 - Dallas 1-Wire(R) master.

- Description: CRC algorithms typically used in a 1-Wire(R) environment.

Revision

1.7

Date

Thursday, August 19, 2004 14:27:16 UTC

### Functions

- unsigned char OWI_CheckRomCRC (unsigned char *romValue)

Calculate and check the CRC of a 64 bit ROM identifier.

- unsigned int OWI_ComputeCRC16 (unsigned char inData, unsigned int seed)

Compute the CRC16 value of a data set.

- unsigned char OWI_ComputeCRC8 (unsigned char inData, unsigned char seed)

Compute the CRC8 value of a data set.

### Function Documentation

#### unsigned char OWI_CheckRomCRC (unsigned char * romValue)

This function computes the CRC8 value of the first 56 bits of a 64 bit identifier. It then checks the calculated value against the CRC value stored in ROM.

Parameters:

romvalue A pointer to an array holding a 64 bit identifier.

Return values:

OWI_CRC_OK The CRC's matched.

OWI_CRC_ERROR There was a discrepancy between the calculated and the stored CRC.

#### unsigned int OWI_ComputeCRC16 (unsigned char inData, unsigned int seed)

This function will compute the CRC16 of inData using seed as inital value for the CRC.

Parameters:

inData One byte of data to compute CRC from.

seed The starting value of the CRC.

Returns:

The CRC16 of inData with seed as initial value.

Note:

Setting seed to 0 computes the crc16 of the inData.

Constantly passing the return value of this function As the seed argument computes the CRC16 value of a longer string of data.

#### unsigned char OWI_ComputeCRC8 (unsigned char inData, unsigned char seed)

This function will compute the CRC8 or DOW-CRC of inData using seed as inital value for the CRC.

Parameters:

inData One byte of data to compute CRC from.

seed The starting value of the CRC.

Returns:

The CRC8 of inData with seed as initial value.

Note:

Setting seed to 0 computes the crc8 of the inData.

Constantly passing the return value of this function As the seed argument computes the CRC8 value of a longer string of data.

## OWIcrc.h File Reference

### Detailed Description

Atmel Corporation

- File: OWIcrc.h

- Compiler: IAR EWAAVR 3.20a

- Support mail: avr@atmel.com

- Supported devices: All AVRs.

- Application Note: AVR318 - Dallas 1-Wire(R) master.

- Description: Header file for OWIUtils.c

Revision

1.7

Date

Thursday, August 19, 2004 14:27:16 UTC

### Functions

- unsigned char OWI_CheckRomCRC (unsigned char *romValue)

Calculate and check the CRC of a 64 bit ROM identifier.

- unsigned int OWI_ComputeCRC16 (unsigned char inData, unsigned int seed)

Compute the CRC16 value of a data set.

- unsigned char OWI_ComputeCRC8 (unsigned char inData, unsigned char seed)

Compute the CRC8 value of a data set.

### Function Documentation

#### unsigned char OWI_CheckRomCRC (unsigned char * romValue)

This function computes the CRC8 value of the first 56 bits of a 64 bit identifier. It then checks the calculated value against the CRC value stored in ROM.

Parameters:

romvalue A pointer to an array holding a 64 bit identifier.

Return values:

OWI_CRC_OK The CRC's matched.

OWI_CRC_ERROR There was a discrepancy between the calculated and the stored CRC.

#### unsigned int OWI_ComputeCRC16 (unsigned char inData, unsigned int seed)

This function will compute the CRC16 of inData using seed as inital value for the CRC.

Parameters:

inData One byte of data to compute CRC from.

seed The starting value of the CRC.

Returns:

The CRC16 of inData with seed as initial value.

Note:

Setting seed to 0 computes the crc16 of the inData.

Constantly passing the return value of this function As the seed argument computes the CRC16 value of a longer string of data.

#### unsigned char OWI_ComputeCRC8 (unsigned char inData, unsigned char seed)

This function will compute the CRC8 or DOW-CRC of inData using seed as inital value for the CRC.

Parameters:

inData One byte of data to compute CRC from.

seed The starting value of the CRC.

Returns:

The CRC8 of inData with seed as initial value.

Note:

Setting seed to 0 computes the crc8 of the inData.

Constantly passing the return value of this function As the seed argument computes the CRC8 value of a longer string of data.

## OWIdefs.h File Reference

### Detailed Description

Atmel Corporation

- File: OWIdefs.h

- Compiler: IAR EWAAVR 3.20a

- Support mail: avr@atmel.com

- Supported devices: All AVRs.

- Application Note: AVR318 - Dallas 1-Wire(R) master.

- Description: Common definitions used for all implementations.

Revision

1.7

Date

Thursday, August 19, 2004 14:27:16 UTC

## OWIDeviceSpecific.h File Reference

### Detailed Description

Atmel Corporation

- File: OWIDeviceSpecific.h

- Compiler: IAR EWAAVR 3.20a

- Support mail: avr@atmel.com

- Supported devices: All AVRs with UART or USART module.

- Application Note: AVR318 - Dallas 1-Wire(R) master.

- Description: Device specific defines that expands to correct register and bit definition names for the selected device.

Revision

1.7

Date

Thursday, August 19, 2004 14:27:16 UTC